DATA AD VOICE COMMUNICATION

FREQUENCY SHIFT KEYING (FSK)

%>>>>>>>>> MATLAB code for binary FSK modulation and de-modulation >>>>>>>%

clc;

clear all;

close all;

x=[ 1 0 0 1 1 0 1];                                    % Binary Information

bp=.000001;                                                    % bit period

disp(' Binary information at Trans mitter :');

disp(x);

%XX representation of transmitting binary information as digital signal XXX

bit=[];

for n=1:1:length(x)

if x(n)==1;

se=ones(1,100);

else x(n)==0;

se=zeros(1,100);

end

bit=[bit se];

end

t1=bp/100:bp/100:100*length(x)*(bp/100);

subplot(3,1,1);

plot(t1,bit,'lineWidth',2.5);grid on;

axis([ 0 bp*length(x) -.5 1.5]);

ylabel('amplitude(volt)');

xlabel(' time(sec)');

title('transmitting information as digital signal');

%XXXXXXXXXXXXXXXXXXXXXXX Binary-FSK modulation XXXXXXXXXXXXXXXXXXXXXXXXXXX%

A=5;                                          % Amplitude of carrier signal

br=1/bp;                                                         % bit rate

f1=br*8;                           % carrier frequency for information as 1

f2=br*2;                           % carrier frequency for information as 0

t2=bp/99:bp/99:bp;

ss=length(t2);

m=[];

for (i=1:1:length(x))

if (x(i)==1)

y=A*cos(2*pi*f1*t2);

else

y=A*cos(2*pi*f2*t2);

end

m=[m y];

end

t3=bp/99:bp/99:bp*length(x);

subplot(3,1,2);

plot(t3,m);

xlabel('time(sec)');

ylabel('amplitude(volt)');

title('waveform for binary FSK modulation coresponding binary information');

%XXXXXXXXXXXXXXXXXXXX Binary FSK demodulation XXXXXXXXXXXXXXXXXXXXXXXXXXXXX

mn=[];

for n=ss:ss:length(m)

t=bp/99:bp/99:bp;

y1=cos(2*pi*f1*t);                    % carrier siignal for information 1

y2=cos(2*pi*f2*t);                    % carrier siignal for information 0

mm=y1.*m((n-(ss-1)):n);

mmm=y2.*m((n-(ss-1)):n);

t4=bp/99:bp/99:bp;

z1=trapz(t4,mm)                                             % intregation

z2=trapz(t4,mmm)                                            % intregation

zz1=round(2*z1/bp)

zz2= round(2*z2/bp)

if(zz1>A/2)      % logic lavel= (0+A)/2 or (A+0)/2 or 2.5 ( in this case)

a=1;

else(zz2>A/2)

a=0;

end

mn=[mn a];

end

disp(' Binary information at Reciver :');

disp(mn);

%XXXXX Representation of binary information as digital signal which achived

%after demodulation XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

bit=[];

for n=1:length(mn);

if mn(n)==1;

se=ones(1,100);

else mn(n)==0;

se=zeros(1,100);

end

bit=[bit se];

end

t4=bp/100:bp/100:100*length(mn)*(bp/100);

subplot(3,1,3)

plot(t4,bit,'LineWidth',2.5);grid on;

axis([ 0 bp*length(mn) -.5 1.5]);

ylabel('amplitude(volt)');

xlabel(' time(sec)');

title('recived information as digital signal after binary FSK demodulation');

%>>>>>>>>>>>>>>>>>>>>>>>>>> end of program >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>%

NOTE: Use Semicolon ';' in order to suppress the output from coming to the MATLAB's Command Window, whenever declaring a periodic pulse, as it can display a vary large matrix in the output, so you can miss what you want.

Post lab questions

- Run the program and explain the shape of the FSK graphs generated.

- Modify the program by the choosing new data pattern and obtain the new results.

- State the advantage of FSK over ASK